REMEMBER FORWARD

TIER D — HASTELLOY C-276 CAPSULE

The chemical specialist. For volcanic, acidic, and industrially contaminated environments.

$600–$1,200 · 10,000+ YEARS · SUPERIOR TO TITANIUM IN CORROSIVE ENVIRONMENTS

Hastelloy C-276 is a nickel-molybdenum-chromium superalloy developed specifically for environments where titanium corrodes. It is the material of choice in nuclear waste containment, chemical processing plants, and deep-sea scientific instruments. In volcanic soils, acidic bogs, sulfurous environments, and ocean trenches, it outperforms every other practical capsule material.

# When to Choose Hastelloy Over Titanium

Titanium Grade 5 is an excellent capsule material in most conditions. There are specific environments where it is not the best choice, and Hastelloy C-276 is the answer in those environments.

| Environment | Titanium Grade 5 | Hastelloy C-276 | Recommendation |
| --- | --- | --- | --- |
| Neutral pH soil, inland | Excellent | Excellent | Either. Titanium cheaper. |
| Acidic soil (pH < 5) | Good, some risk | Excellent | Hastelloy preferred |
| Volcanic / sulfurous soil | Moderate risk from H₂S | Excellent | Hastelloy strongly preferred |
| Seawater / ocean floor | Good with gold plate | Excellent, no plating needed | Hastelloy preferred |
| Hydrofluoric acid environments | Corrodes | Excellent | Hastelloy only |
| Reducing acids (HCl, H₂SO₄) | Moderate resistance | Excellent resistance | Hastelloy preferred |
| Neutral coastal soil | Good | Excellent | Either acceptable |
| Freshwater burial | Excellent | Excellent | Either. Titanium cheaper. |

# Material Properties

## Chemical Composition

| Element | Percentage | Role |
| --- | --- | --- |
| Nickel (Ni) | 57% (balance) | Base metal, corrosion resistance foundation |
| Molybdenum (Mo) | 16% | Resistance to reducing acids (HCl, H₂SO₄), pitting resistance |
| Chromium (Cr) | 15.5% | Oxidizing acid resistance, general corrosion resistance |
| Iron (Fe) | 5.5% | Structural |
| Tungsten (W) | 3.7% | Additional pitting and crevice corrosion resistance |
| Cobalt (Co) | 2.5% max | Structural |
| Carbon (C) | 0.01% max | Low carbon prevents sensitization at weld zones |

## Key Performance Properties

- Resists hydrochloric acid at all concentrations and temperatures

- Resists sulfuric acid including hot concentrated solutions

- Resists hydrofluoric acid — one of the most aggressive chemicals in existence

- Resists hydrogen sulfide (H₂S) — the primary corrosive agent in volcanic environments

- No pitting, no crevice corrosion, no stress corrosion cracking in seawater

- Operating temperature range: cryogenic to 1,080°C

- Density similar to other nickel alloys: 8.89 g/cm³ (vs titanium 4.43 g/cm³ — approximately twice as heavy)

# Design Specification

The Hastelloy capsule uses the same design architecture as the titanium sphere, with the alloy substituted throughout. All internal specifications are identical.

| Component | Specification | Purpose |
| --- | --- | --- |
| Outer shell | Hastelloy C-276, 6" sphere, 3mm wall thickness | Primary chemical and mechanical protection |
| O-rings | Kalrez (FFKM) preferred, Viton (FKM) acceptable | Kalrez outperforms Viton in aggressive chemical environments |
| Inner frame | Octahedron geometry, Hastelloy C-276 or Grade 316L SS | Load distribution and plate protection |
| Pyramid cavity | Center of chamber, apex at 51.83° Giza angle | Geometric preservation feature |
| Atmosphere | Argon gas fill | Prevents oxidation of plate surfaces |
| Insulation | Silica aerogel lining | Thermal buffering, moisture exclusion |
| Fasteners | Hastelloy C-276 or Grade 316L stainless | No galvanic mismatch between dissimilar metals |

## Important: Avoid Galvanic Corrosion

Hastelloy C-276 must not be in direct metal-to-metal contact with titanium, aluminum, or carbon steel inside a moist environment. Galvanic couples between dissimilar metals accelerate corrosion significantly. Use only Hastelloy, high-grade stainless steel (316L), or non-metallic components for all interior hardware. This is especially critical in the O-ring retaining hardware and lid fasteners.

# Sourcing and Cost

Hastelloy C-276 is a specialty alloy and is not available at hardware stores. It must be sourced from specialty metal suppliers. Cost is approximately 3 to 5 times that of equivalent titanium by weight.

| Item | Estimated Cost | Source |
| --- | --- | --- |
| Hastelloy C-276 sphere blank or machined sphere | $400–$800 | Specialty alloy fabricators: ATI, Haynes International, online metal suppliers |
| Kalrez O-rings (set) | $50–$150 | Parker Hannifin, specialty seal suppliers |
| Hastelloy fasteners | $30–$80 | Specialty fastener suppliers |
| Argon fill (small cylinder) | $30–$60 | Welding supply store |
| Silica aerogel blanket | $20–$50 | Industrial insulation suppliers |
| TOTAL ESTIMATE | $530–$1,140 |  |

# Where to Deploy This Design

- Iceland, Hawaii, Canary Islands, Japan — volcanic soils with high sulfur content

- Scottish Highlands, Scandinavia, Canada — acidic peat bog environments

- Industrial brownfield sites — contaminated soils with chemical residues

- Ocean floor deployment at any depth — no additional gold plating required

- Coastal salt marsh environments

- Any location where soil pH testing shows values below 5.0

# Soil pH Testing Before Burial

Before choosing between titanium and Hastelloy, test the burial site soil pH. Simple litmus paper or inexpensive pH test kits (available at garden centers) give sufficient accuracy.

- pH 7.0 (neutral): either titanium or Hastelloy suitable

- pH 6.0–7.0 (mildly acidic): titanium suitable with minor long-term risk

- pH 5.0–6.0 (moderately acidic): Hastelloy preferred

- pH below 5.0 (strongly acidic): Hastelloy required for 1,000+ year survival

- Sulfur smell present: Hastelloy required regardless of pH reading

Remember Forward — The Patient Message

Buy it. Build it. Bury it. For someone you will never meet.

CC BY-SA 4.0 — All materials free to use, share, and adapt.